On April 19, 2017 Jersey City launched an initiative titled “Year of Water for Jersey City“, which used the FEMA flood zone maps to determine that 40 percent of Jersey City is within a FEMA flood zone (Year of Water press Release, 2017).

Additionally, see the maps below that we used to form discussion around the idea of storm water management and resiliency:

2050 SPECIAL FLOOD HAZARD AREAS:

This map helped our team because we were able to discuss how sea level rise would influence different parts of the city under four levels of sea level rise projections. We noticed that the majority of the downtown area will be submerged and a large concern to the team was the submergence of the Holland Tunnel and I-78.

SLOSH Category 4

Jersey City is significantly impacted under SLOSH categories 1, 2, 3, & 4.  The implications of these maps are that Jersey City must make sustainable initiatives related to storm water management and flood resiliency a priority.  In any of the four scenarios, Jersey City suffers a loss of major transportation facilities and the loss of law enforcement facilities, fire stations, and hospitals.

SEA LEVEL RISE @ 1 FOOT:

The team analyzed the location of key facilities and their susceptibility to damage under a 1-foot sea level rise. The team noticed that access to parks, industrial facilities in the south, and NJ Transit facilities will incur inundation.

SEA LEVEL RISE @ 2 FEET:

Under the two-foot sea level rise scenario we noticed a concerning rise in water around the Jersey City Medical Center located at the corner of Grand Street and Jersey Ave. This facility is important for its Urgent Care operation. Additionally, we noticed that Light Rail facilities in the South and NJ transit facilities in the North would suffer flood damage.

SEA LEVEL RISE @ 3 FEET:

In the 3-foot sea level rise scenario, our team saw that the Jersey City Medical center and the NJ Transit Rail facilities would likely suffer heavy damage.  The Medical Center and Rail Station are of particular concern because in a natural disaster scenario there needs to be public transit that can take people to safe places and there needs to be medical facilities that treat people with injuries. These areas are being addressed by the City for flood mitigation.

MARSH MIGRATION MAP @ 3 FEET SEA LEVEL RISE:

We noticed that Jersey City’s Marsh habitats are concentrated in Liberty State Park and in Lincoln Park. Jersey City prides itself in prioritizing open space and we wish to further preserve our marsh habitats. We were relieved to see that the marsh habitat in Lincoln Park and Liberty Park is not significantly impaired by a 3-foot sea level rise.

The following Maps were made using the GTR Tool Map Maker:

This Map shows the “Sandy Surge” and an outline around Jersey City.  The team recalled how devastating Hurricane Sandy was and this map solidified the urgency for climate action. Specifically, the flooded areas engulfed major transportation nodes (i.e. I-78, HBLR, Exchange Place, the Hoboken Terminal, Grove Street Path, Newport Path, and Portions of 1 & 9) and key facilities (i.e. Hospitals, Police Departments, Fire Stations, senior homes, etc.).

This map shows evacuation routes highlighted in yellow. The identified the Palisades ridge, which e=serves as a natural barrier to floods.  The arterial road that passes through Bayonne into Staten Island was identified as a good evacuation route.

This map shows the concentration of Businesses by census tracts.  The darker the red the higher the concentration. The team was concerned to see that when sea levels rose above 6ft Jersey City’s highest concentration of businesses in the Downtown district are adversely affected.  The economic implications of a flood that damages business would be detrimental to the city.

3. Completing the on-line Getting to Resilience: Community Planning Evaluation Tool (hereafter referred to as the GTR Tool).

See Attachment

Step 3: Armed with the knowledge gained from the NJ Flood Mapper Tool, use the Getting To Resilience (GTR) Tool to determine your community’s resiliency to flooding concerns.

Our team determined that we need to do much more to protect our waterfront.  The highest concentration of Jersey City residents is in the Downtown area, and this is also the most adversely influenced area by any flooding events.  Jersey City needs to incorporate the deficiencies it has in policies that were identified by the GTR Tool:

High Water Mark Signs, Annual Outreach to Flood Plain Residents, Communicate with Property Owners to inform them of what they need to prepare for a flood disaster, Conduct Outreach with Jersey’s special Needs registry, inform property owners of FEMA suggested means to protect their homes, etc. (See the attachment for the remaining suggestions)

Step 4: Submit the results of the Flood Mapper exercise and Getting to Resilience to the municipal Planning Board and Environmental Commission, if one exists.

These products must be provided to all members of the Planning Board and the Environmental Commission (if one exists).